FlyingRC® Mini BEC Step-Down Module For DJI O4

Product Manual

Product Dimensions Diagram

- Technical Specifications

- Input Voltage: 7–36V (2–8S LiPo)

- Output Voltage: 5V/2A; 9V/2A. Default output: 5V; 9V selected by soldering the jumper.

- Dimensions：8.4mm × 17.1mm × 6.3mm

- Weight：1g

- Package Contents: 1× module, 1× heat shrink tube

- Product Features

- Compatible with DJI O3 / O4 / O4 PRO digital VTX

- Direct plug-and-play connection with factory-standard pinout.

- Input Voltage: 7–36V, 2–8S LiPo (The Input voltage must be 0.8V higher than the set output voltage; otherwise, the output voltage will be 0.8V lower than the input voltage.)

- Output Voltage: 5V / 9V selectable. Default output: 5V; 9V can be selected via jumper pads.

- Continuous Output Current:5V/2A; 9V/2A

Front View

Rear View

- 3. User Instructions

- Input & Output Connection Diagram

FC side: Connect to a flight controller without BEC;

DJI side: Connect to DJI Air Unit

- Select output voltage via jumper pads. Default output is 5V when jumper pads open.

Short the 9V jumper pad

- 3. Short the jumper pad to set output voltage to 9V.

Other FlyingRC® Products

| - FlyingRC® F4WSE Pro - F405 Fixed-Wing Flight Controller Upgraded Version ¦ Enhanced Performance Manual：https://www.kdocs.cn/l/cavm9EoVEq5A |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - FlyingRC® F4WSE MK1.5 - F405 Fixed-Wing Flight Controller Classic model, unique design, highly acclaimed. Manual：https://kdocs.cn/l/cpyd8MTS56DD |  |  |  |  |  |  |  |
| - FlyingRC® H7Wlite MK1 H743 Fixed-Wing Flight Controller Flagship performance, full expansion, high-power BEC. Manual：https://kdocs.cn/l/cpYaBiFKh8U1 |  |  |  |  |  |  |  |  |
| - FlyingRC® F4Wing Mini MK1 F405 Fixed-Wing Flight Controller 2025 bestseller, ultra-compact, smallest on the market, great value. Manual：https://kdocs.cn/l/cdUIdmpSpW7J |  |  |  |  |  |  |  |  |
| - FlyingRC® AM32 Dual ESC 40A 2-in-1 ESC Unique design for dual-power aerial, land and water vehicles including drones. Manual：https://www.kdocs.cn/l/cjwNtsKK6HeR |  |  |  |  |  |
| - FlyingRC® AM32 ESC V2.5 75A - Single ESC Infineon gold-package MOSFETs, premium materials, superior heat dissipation. Manual：https://kdocs.cn/l/cv6hUy67IdTJ |  |  |  |  |  |  |  |
| - FlyingRC® AM32 Mini ESC 40A V1.0 - Single ESC Ultra-miniature, ideal for micro drones, high over-current capability, stable operation. Manual：https://kdocs.cn/l/cqdLSysEyWFh |  |  |  |  |  |  |  |
| - FlyingRC® 10A 12S 440A Power Distribution Board for FPV Drones Industry top choice, excellent quality and affordable price. Manual：https://kdocs.cn/l/coel7OgrlCBP |  |  |  |  |  |  |  |
| - FlyingRC® H7D MK1 H743 FPV Flight Controller with Dual Gyro Powerful computing, stable and precise flight, rich interfaces, smooth control. Manual：https://kdocs.cn/l/cqMh1I0RwAoD |  |  |  |  |  |  |  |
| - FlyingRC® F420 MK1 F405 FPV Flight Controller (20/30.5mm Mounting) Dual mounting holes, all connectors, no soldering required, compliant with Betaflight interface standards. Manual：https://kdocs.cn/l/ctGeLiN9ZYKj |  |  |  |  |  |  |  |
| - FlyingRC® 4-in-1 75A V3.2 Gold-Sealed FPV ESC Infineon gold-package MOSFETs, superior heat dissipation & over-current capability, premium choice. Manual：https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |
| - FlyingRC® Stack Combo: H743 Flight Controller + 4-in-1 75A Gold-Sealed ESC Professional flight choice, flagship materials & craftsmanship, reasonable price. Manual：https://kdocs.cn/l/cqMh1I0RwAoD https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: F405 Flight Controller + 4-in-1 75A Gold-Sealed ESC Best-selling combo, excellent performance, affordable price. Manual：https://kdocs.cn/l/ctGeLiN9ZYKj https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |
| - FlyingRC® ELRS 2.4G Diversity Receiver True diversity reception, temperature compensation, high-sensitivity reception. Manual：https://kdocs.cn/l/cbvlOlShmsPS |  |  |  |  |  |
| - FlyingRC® 10A 12S BEC Step-Down Module Wide voltage input, high power, compact size, multiple voltages available, 10A continuous output. Manual：https://kdocs.cn/l/clssNCKgNhLV |  |  |  |  |  |
| - FlyingRC® 5A BEC Step-Down Module Ultra-small size, multiple voltages available, 5A continuous output. Manual：https://kdocs.cn/l/cb9VdNywqCKA |  |  |  |  |  |
| - FlyingRC® Mini BEC For DJI O4 Easy wiring, ultra-small size, stable power supply. Manual：https://www.kdocs.cn/l/ckkebieur37K |  |  |  |  |  |
| - FlyingRC® CAN Bus Serial & PWM Expansion Board Long-distance transmission, high speed, excellent stability. Manual：https://www.kdocs.cn/l/cmxRyBnU4BXj |  |  |  |  |  |
| - FlyingRC® AM3 2ESC Programmer Supports BL, BL32, AM32. Simple and easy to use. Manual：https://kdocs.cn/l/cogBSEBe67mE |  |  |  |  |  |
| - FlyingRC® Digital Airspeed Sensor without Pitot Tube Integrated design, smallest on the market. MS4525D protocol, I2C interface. Manual：https://www.kdocs.cn/l/clPvEEEcJgCC |  |  |  |  |  |
| - FlyingRC® U-Blox M10 GPS Compatible with all open-source flight controllers. Manual：https://www.kdocs.cn/l/cdSYQrzx74O8 |  |  |  |  |  |