FlyingRC® F4D MK1 F405 FPV Drone Flight Controller (20/30.5mm Mounting) Product Manual

FlyingRC® is a globally-oriented professional brand of electronic devices. We specialize in the R&D and sales of drone electronics, core robot circuits and sensors. Our design team is based in Singapore, and our products are sold worldwide.

Since launched in 2023, FlyingRC® products have gained widespread recognition from customers as well as strong market response. As of April 2026, our cumulative sales volume has exceeded 46,000 units. Our products are widely applied in industries, robotics, emergency rescue, environmental protection & inspection, aerial photography, public security, retail and many other fields. Above all, we are committed to distinctive design and superior reliable quality.

Our Chief Technology Officer Liu Yuhan possesses outstanding expertise in circuit design. Over the past four years, he has designed more than 80 circuit projects and contributed 39 to open-source platforms. Moreover, he won the Design Award on the open-source platform X.JLC.com twice in 2023 and 2024, and has become the youngest certified designer on this platform.

He holds the NUS (National University of Singapore, rank 8th worldwide) Science and Technology Scholarship, and is currently majoring in Electronic Engineering with a focus on Aerospace Technology. In addition, all members of our technical team are graduates from prestigious universities with extensive professional product experience.

- Technical Specifications

- MCU: STM32F405RGT6; Gyroscope: ICM-42688-P; High-Precision Barometer: Goertek SPA06-003

- Analog/HD OSD，6xUARTs（only RX pin of UART3 is exposed），5xPWMs（4 ESC + 1LED），1xI2C，1xADC（Current Sensor），Onboard 5V3A/9V2A BEC

- Black Box: Onboard 16MB NOR Flash

- 7-28V DC Input (2-6S LiPo. Note: When using 2S or 3S LiPo batteries, the input voltage to the VTX BEC must be above 9.8V for stable output; 3S-6S LiPo is strongly recommended)

- Dual BEC(MP9943 + MP9943) : 5V 3A (for Devices) & 9V 2A (for VTX and Camera)

- Dimensions & Weight：36.6mm*36.6mm*8.0mm，6.3g， 28.1mm*31.1mm*8.0mm (after trimming mounting tabs)

Package Contents:

- 1. 1 x FlyingRC F4D F405 Flight Controller

- 2. 4 x M3 Silicone Anti-Vibration Standoffs

- 3. 1 x 15cm DJI VTX 6-Pin Cable

- 4. 1 x 10cm SH1.0 4-Pin Ultra-Soft Silicone Cable

- 5. 1 x SH1.0 8-Pin Crimp Terminal

- 6. 1 x 3.5cm SH1.0 8-Pin 30AWG Ultra-Soft Silicone Cable

- 7. 4 x M2 Silicone Anti-Vibration Standoffs

- Product Features

- Equipped with STM32F405RGT6 MCU, supporting AP, BF, and INAV firmware; pre-flashed with BF firmware at factory.

- InvenSense 3rd-generation ICM-42688-P gyroscope offers higher accuracy and stability, optimized specifically for drones.

- Built-in high-precision Goertek SPA06-003 barometer.

- Compatible with 30.5mm M4 standard mounting holes (fits M3 screws with Anti-Vibration Standoffs) and 20mm M4 mounting holes (fits M2/M3 screws with Anti-Vibration Standoffs), supporting 2–12 inch FPV drones with excellent frame compatibility.

- 4+1 PWM outputs supporting 4-in-1 ESCs, 6 UARTs (only RX pin of UART3 exposed), and 1 I2C, providing rich interfaces.

- Full SH1.0 socket design: ESCs, receivers, GPS, analog VTX, analog cameras, HD VTX, and LED strips all connect via connectors, eliminating the difficulty of soldering small pads and solder bridging.

- Onboard 9V BEC for VTX and camera, switchable via PINIO1 (User1 in BF firmware), preventing VTX from overheating and burnout during bench testing.

- Onboard 16MB Flash ensures sufficient storage for multiple flight log records.

- Black solder mask, 1OZ copper thickness, 1μm immersion gold, resin plug holes, and via-in-pad technology. Assembled with JLCPCB SMT service for reliable and superior quality.

| Basic Parameters |  | Sensors |  |
| --- | --- | --- | --- |
| Model | FlyingRC® F4D | IMU(Gyro & Accelerometer) | Invensense Gen 3 ICM-42688-P |
| Dimensions | 36.6*36.6*8.0 28.1*31.1/*8.0mm (after cutting) | Barometer | Goertek SPA06-003 |
| Mounting Pattern | 30.5 / 20 mm | Magnetometer | No onboard magnetometer |
| Weight | 6.3g | Analog OSD | AT7465E |
| MCU |  | Power Output |  |
| MCU Chip | STM32F405RGT6 | BEC Chip Model | MP9943+MP9943 |
| Clock Speed | 168 MHz | Onboard BEC (Devices) | 5V3A |
| Flash | 1 MB | Onboard BEC (VTX) | 9V2A |
| RAM | 192 KB |  |  |
| Interfaces |  | Firmware Support |  |
| UART | 6 UARTs | Betaflight | Supported (factory default) |
|  | UART3: RX only | INAV | Supported |
| PWM | 5 channels (including 1 LED channel) | Ardupilot | Supported |
| I2C | 1 | PX4 | Not supported |
| Current ADC Sampling | Supported |  |  |
| SWD Debug | Not available | Operating Environment |  |
| Buzzer Port | None | VBAT Input Range | 7-28V DC IN 2-6S LiPo |
| LED Strip Port | Supports WS2812 (requires soldering) | Power Input | Same as VBAT input range |
| USB-TYPE-C | Onboard direct plug | Operating Temperature | -10-100℃ |
| Blackbox Storage | Onboard 16MB NOR Flash | Storage Temperature | 0-40℃ |
| SBUS | Supported |  |  |

- 3. Usage Instructions

LAYOUT：

3D Diagram of the Flight Controller - Front Side

3D Diagram of the Flight Controller - Reverse Side

Connector Function Diagram - Front Side

Connector & Pad Function Diagram - Reverse Side

- Pre-Use Preparation

- Note: Due to varying wire sequences between flight controllers and ESCs from different manufacturers, ensure the 8P ESC connection cable is made with the correct wire sequence matching your ESC. For wire sequence inquiries, please consult our customer group. Incorrect wire sequencing could result in the flight controller being burned out!!!

- This product adopts the latest official Betaflight pin order. Reference: Betaflight Connector Standard (official documentation page)

- When inserting the 8P female header, ensure both ends are perfectly parallel during insertion. Otherwise, edge pins may bend or be pushed out, resulting in socket failure!

Wiring Diagram (Applicable to BF, AP, and INAV firmware)：

Flight Controller Wiring Diagram

(Note: The ESC is V3.2 Version)

SH1.0 female connectors on the flight controller are marked with silkscreen labels.

The ESCs are currently available in two versions: V3.2 and V4.0, with different wiring orders.

V4.0 complies with the Betaflight (BF) wiring standard. Please note the visual differences between V3.2 and V4.0.

1.The 8P connector on V4.0 is marked with “Note Wiring Order”.

V3.2                           V4.0

2.The version number is labeled on both variants.

3.V4.0 is marked with a serial number (S/N).

V3.2                           V4.0

- ESC Connection

Connection with FlyingRC® 4IN1 75A Gold-Sealed ESC

(Note: The ESC is V3.2 Version)

Connection with FlyingRC® 4IN1 75A Gold-Sealed ESC

(Note: The ESC is V4.0 Version)

- ELRS Receiver

- Note the TX/RX cross-connection: Flight controller TX → Receiver RX; Flight controller RX → Receiver TX

- Connection with FlyingRC® ELRS Receiver

- GPS

- Connection with FlyingRC® GPS

- Digtital VTX

- Connection to Digital Video Transmission (SBUS supported on F4D Flight Controller)

- Analog Camera and VTX

- Connection with Analog Camera and VTX

- Firmware Flashing of the Flight Controller

BF, AP, and INAV firmwares are recommended. The factory default firmware is BF.

The following wiring instructions use BF firmware as an example.

Download links for the latest stable version (as of 2025/06/15)：

ArduPlane4.6.1	Matek F405-TE	ArduPlane4.6.1

Matek F405-TE(Download）

ArduCopter4.6.1	Matek F405-TE	ArduCopter4.6.1

Matek_F405-TE(Download）

INAV: Matek F405TE

BF:

BF Firmware Flashing Tutorial: Bilibili Column ---- Ardupilot Fixed-Wing - Flight Controller Firmware Flashing and Version Update

Please note: Limited technical support is provided for BF and Ardupilot; no technical support is available for INAV.

Detailed Explanation of the Flight Controller Interfaces ：

- UARTs（UARTx 、Serialx）

!!! Note: In Ardupilot firmware, Serial numbers do not correspond one-to-one with UART/USART numbers. Refer to the correspondence table below. !!!

| Ardupilot UART Mapping |  |  |  |  |
| --- | --- | --- | --- | --- |
| PCB Silkscreen | UART Number | Config | Protocol | SERIAL_X |
| USB | USB | USB |  | SERIAL0 |
| DJI | UART1 | telem1 | with DMA | SERIAL1 |
| U2 | UART2 | RC input/Receiver | CRSF | SERIAL6 |
| ESC (R3) | UART3 | ESC Telemetry | NO DMA | SERIAL2 |
| U4 | UART4 | User Define | NO DMA | SERIAL4 |
| GPS | UART5 | GPS | NO DMA | SERIAL3 |
| U6 | UART6 | User Define | TX6 with DMA | SERIAL5 |

Ardupilot UART Correspondence Table

!!! Note: After wiring, the flight controller parameters must be adjusted accordingly. For example, in the above sample diagram, if the GPS is connected to R2 and T2, you can find in the UART correspondence table that R2 and T2 correspond to Serial3. In this case, set Serial3_Protocol to GPS, and ensure that the function of other ports is not set to GPS. !!!

If using receivers with non-SBUS/PPM protocols such as ELRS on Serial7, set brd_alt_config to 1.

- PWM Output Functions

| PWM Channels |  |  |  |  |
| --- | --- | --- | --- | --- |
| PWM Group | PWM Channels | GPIO | Timer | DMA/DShot |
| Group1 | S1 | PWM1 GPIO50 | TIM8_CH4 | DMA/DShot |
|  | S2 | PWM2 GPIO51 | TIM8_CH3 | DMA/DShot |
| Group2 | S3 | PWM3 GPIO52 | TIM1_CH3N | DMA/DShot |
|  | S4 | PWM4GPIO53 | TIM1_CH1 | DMA/DShot |

PWM Output Function Table

PWM ports in the same group cannot be used simultaneously for Dshot ESCs and 50Hz PWM servos

- I2C Bus

| I2C |  |  |  |
| --- | --- | --- | --- |
| I2C number | Config | Parameterl | Value |
| I2C1 | Compass | COMPASS_AUTODEC | 1 |
|  | onboard Baro DPS310 / DSP368 | Address | 0x76 |
|  | Digital Airspeed I2C | ARSPD_BUS | 1 |
|  | MS4525 | ARSPD_TYPE | 1 |
|  | DLVR-L10D | ARSPD_TYPE | 9 |

- I2C Bus Table

- ADC Analog Signal Inputs

| ADC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin | Voltage Tolerance | Definition | Config | Value |
| Vbat | 1K:20K divider builtin | on board battery voltage | BATT_VOLT_PIN | 14 |
|  | 0~28V |  | BATT_VOLT_MULT | 21 |
| Curr | 0~3.3V | on board current sensor | BATT_CURR_PIN | 15 |
|  |  |  | BATT_AMP_PERVLT | 66.7 |

ADC Analog Signal Input Table

Betaflight General Parameter Settings:

Gyro installation orientation: CW90Flip

Ardupilot General Parameter Settings:

LOG_BACKEND_TYPE = 4 (enables TF card black box function to record flight logs)

BATT_VOLT_PIN     10

BATT_VOLT_MULT   11.0，(Default voltmeter scaling factor, with a 5% error margin; additional calibration is unnecessary.)

BATT_CURR_PIN     11

INAV General Parameter Settings:

Default voltmeter scaling factor 1100

- Other FlyingRC® Products

| - FlyingRC® F4WSE Pro - F405 Fixed-Wing Flight Controller - Upgraded Version ¦ Enhanced Performance - Manual：https://www.kdocs.cn/l/cavm9EoVEq5A |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - FlyingRC® F4WSE MK1.5 - F405 Fixed-Wing Flight Controller - Classic model, unique design, highly acclaimed. - Manual：https://kdocs.cn/l/cpyd8MTS56DD |  |  |  |  |  |  |  |  |
| - FlyingRC® H7Wlite MK1 H743 Fixed-Wing Flight Controller - Flagship performance, full expansion, high-power BEC. - Manual：https://kdocs.cn/l/cpYaBiFKh8U1 |  |  |  |  |  |  |  |  |
| - FlyingRC® F4Wing Mini MK1 F405 Fixed-Wing Flight Controller - 2025 bestseller, ultra-compact, smallest on the market, great value. - Manual：https://kdocs.cn/l/cdUIdmpSpW7J |  |  |  |  |  |  |  |  |
| - FlyingRC® AM32 Dual ESC 40A 2-in-1 ESC - Unique design for dual-power aerial, land and water vehicles including drones. - Manual：https://www.kdocs.cn/l/cjwNtsKK6HeR |  |  |  |  |  |  |  |  |
| - FlyingRC® AM32 ESC V2.5 75A - Single ESC - Infineon gold-package MOSFETs, premium materials, superior heat dissipation. - Manual：https://kdocs.cn/l/cv6hUy67IdTJ |  |  |  |  |  |  |  |  |
| - FlyingRC® AM32 Mini ESC 40A V1.0 - Single ESC - Ultra-miniature, ideal for micro drones, high over-current capability, stable operation. - Manual：https://kdocs.cn/l/cqdLSysEyWFh |  |  |  |  |  |  |  |  |
| - FlyingRC® 10A 12S 440A Power Distribution Board for FPV Drones - Industry top choice, excellent quality and affordable price. - Manual：https://kdocs.cn/l/coel7OgrlCBP |  |  |  |  |  |  |  |  |
| - FlyingRC® H7D Pro MK1 H743 FPV Flight Controller with Dual Gyro - Fully Upgraded & More Powerful - Manual：https://www.kdocs.cn/l/ctcekCy676Ez |  |  |  |  |  |  |  |  |
| - FlyingRC® H7D MK1 H743 FPV Flight Controller with Dual Gyro - Powerful computing, stable and precise flight, rich interfaces, smooth control. - Manual：https://kdocs.cn/l/cqMh1I0RwAoD |  |  |  |  |  |  |  |  |
| - FlyingRC® F420 MK1 F405 FPV Flight Controller (20/30.5mm Mounting) - Dual mounting holes, all connectors, no soldering required, compliant with Betaflight interface standards. - Manual：https://kdocs.cn/l/ctGeLiN9ZYKj |  |  |  |  |  |  |  |  |
| - FlyingRC® 4-in-1 75A V3.2 Gold-Sealed FPV ESC - Infineon gold-package MOSFETs, superior heat dissipation & over-current capability, premium choice. - Manual：https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: H743 Flight Controller + 4-in-1 75A Gold-Sealed ESC - Professional flight choice, flagship materials & craftsmanship, reasonable price. - Manual：https://www.kdocs.cn/l/ctcekCy676Ez - https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: F405 Flight Controller + 4-in-1 75A Gold-Sealed ESC - Best-selling combo, excellent performance, affordable price. Manual：https://kdocs.cn/l/ctGeLiN9ZYKj https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: H743 Flight Controller + 4-in-1 45A ESC Winning combo: great performance, budget-friendly price Manual：https://www.kdocs.cn/l/ctcekCy676Ez https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: F405 Flight Controller + 4-in-1 45A ESC Hit combo, great value, budget-friendly Manual：https://kdocs.cn/l/ctGeLiN9ZYKj https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: F4WSE Pro - F405 Fixed-Wing Flight Controller + AM32 Dual ESC 40A 2-in-1 ESC Unique Design • Exclusive Offer • Bestselling Bundle Manual：https://www.kdocs.cn/l/cavm9EoVEq5A https://www.kdocs.cn/l/cjwNtsKK6HeR |  |  |  |  |  |  |
| - FlyingRC® ELRS 2.4G Diversity Receiver - True diversity reception, temperature compensation, high-sensitivity reception. - Manual：https://kdocs.cn/l/cbvlOlShmsPS |  |  |  |  |  |  |  |  |
| - FlyingRC® RM3100 SPI Module 罗盘模块 - Ultra-high Resolution • Low Power • Industry Top Choice - Manual：https://www.kdocs.cn/l/cdGuHvBVyndC |  |  |  |  |  |  |  |  |
| - FlyingRC® 10A 12S BEC Step-Down Module - Wide voltage input, high power, compact size, multiple voltages available, 10A continuous output. - Manual：https://kdocs.cn/l/clssNCKgNhLV |  |  |  |  |  |  |  |  |
| - FlyingRC® 5A BEC Step-Down Module - Ultra-small size, multiple voltages available, 5A continuous output. - Manual：https://kdocs.cn/l/cb9VdNywqCKA |  |  |  |  |  |  |  |  |
| - FlyingRC® Mini BEC For DJI O4 - Easy wiring, ultra-small size, stable power supply. - Manual：https://www.kdocs.cn/l/ckkebieur37K |  |  |  |  |  |  |  |  |
| - FlyingRC® CAN Bus Serial & PWM Expansion Board - Long-distance transmission, high speed, excellent stability. - Manual：https://www.kdocs.cn/l/cmxRyBnU4BXj |  |  |  |  |  |  |  |  |
| - FlyingRC® AM3 2ESC Programmer - Supports BL, BL32, AM32. Simple and easy to use. - Manual：https://kdocs.cn/l/cogBSEBe67mE |  |  |  |  |  |  |  |  |
| - FlyingRC® Digital Airspeed Sensor without Pitot Tube - Integrated design, smallest on the market. MS4525D protocol, I2C interface. - Manual：https://www.kdocs.cn/l/clPvEEEcJgCC |  |  |  |  |  |  |  |  |
| - FlyingRC® U-Blox M10 GPS - Compatible with all open-source flight controllers. - Manual：https://www.kdocs.cn/l/cdSYQrzx74O8 |  |  |  |  |  |  |  |  |