FlyingRC® F4WSE Pro Flight Controller Product Manual

FlyingRC® is a globally-oriented professional brand of electronic devices. We specialize in the R&D and sales of drone electronics, core robot circuits and sensors. Our design team is based in Singapore, and our products are sold worldwide.

Since launched in 2023, FlyingRC® products have gained widespread recognition from customers as well as strong market response. As of April 2026, our cumulative sales volume has exceeded 46,000 units. Our products are widely applied in industries, robotics, emergency rescue, environmental protection & inspection, aerial photography, public security, retail and many other fields. Above all, we are committed to distinctive design and superior reliable quality.

Our Chief Technology Officer Liu Yuhan possesses outstanding expertise in circuit design. Over the past four years, he has designed more than 80 circuit projects and contributed 39 to open-source platforms. Moreover, he won the Design Award on the open-source platform X.JLC.com twice in 2023 and 2024, and has become the youngest certified designer on this platform.

He holds the NUS (National University of Singapore, rank 8th worldwide) Science and Technology Scholarship, and is currently majoring in Electronic Engineering with a focus on Aerospace Technology. In addition, all members of our technical team are graduates from prestigious universities with extensive professional product experience.

- Description:

- Technical Specifications

| Basic specifications |  | Sensor |  |
| --- | --- | --- | --- |
| Model | FlyingRC® F4WSE Pro | IMU(Gyroscope and Accelerometer) | ICM-42688-P |
| Dimensions | 22.6mm*42.8mm*9mm | Barometer | SPA06-001 |
| Weight | 9.1g | Magnetometer | No on-board magnetometer |
|  |  | Simulated OSD | AT7465E |
| MCU |  | Power output |  |
| MCU | STM32F405RGT6 | BEC | MP9943+MP9447 |
| Frequency | 168 MHz | BEC-Device | 5V3A |
| Flash | 1 MB | BEC-Servo | 5V5A |
| RAM | 192 k |  |  |
| Connector |  | Firmware Support |  |
| UART | 6xUART | Betaflight | Supported |
|  | UART1, UART2, UART3 | INAV | Supported |
|  | UART4, UART5, UART6 | Ardupilot | Supported (Factory Default) |
| PWM | A total of 12 channels: 6 channels via 2.54mm pin headers, 2 channels via solder pads, and 4 channels via the 4-in-1 ESC port. | PX4 | Not Supported |
| I2C | 1 |  |  |
| Current-ADC | 80A(cont) /120A(inst) |  |  |
| SWD | N/A | Working Environment |  |
| Buzzer | Supports passive buzzer | VBAT Support range | 7-28V DC IN 2-6S LiPo |
| LED Strip | Supports WS2812 LED strips. When using INAV firmware, connect the LED strip signal port to the S12 solder pad. | Input | Same as VBAT range |
| USB-TYPE-C | External USB Brd | Working Temp | -10-100℃ |
| Blackbox | External TF(Recommended usable capacity：4GB, Maximum supported capacity: 32GB) | Storing Temp | 0-40℃ |
| SBUS | Built-in inverter |  |  |
|  | SBUS is internally connected to the RX pin of UART2. |  |  |

    - Complete upgrade

- Compared to the F4WSE, the F4WSE Pro edition features the following enhancements:

- 1. Upgraded Boot button for improved usability

- 2. USB sub-board requires no soldering

- 3. Supports direct plug-in of 4-in-1 ESCs for easy quadcopter VTOL construction

- 4. Supports direct plug-in connection for receivers, GPS modules and high-definition air units.

- 5. Added dual analogue camera switching function

- 6. PCB upgraded to 2oz copper thickness for improved continuous current handling capability

- Product Features

- Compact design, ideal for small FPV aircraft or modified hand-launch planes.

- Innovative design with strong expandability, supports Y3 or quadcopter aircraft. Equipped with 12 PWM channels, also compatible with large carrier drones and VTOL fixed-wing aircraft.

- Adopts high-precision gyroscopes and barometers, delivering significantly enhanced stability compared to comparable flight controllers.

- Onboard current sensor and built-in dual 5V BEC, providing the strongest BEC current output capability among flight controllers in the same form factor.

- External USB and TF card modules allow more flexible installation layout.

- Supports Ardupilot and INAV firmware.

- Supports LED strip connection for spectacular visual effects during flight.

- LAYOUT:

3D Diagram - Top

3D Diagram - Bottom

Definition Diagram for the Flight Controller 's Direct Plug-in Ports

    - 4.Usage Instructions:

- (1) Preparation before using：

- Preparation of Tools and Materials:

- soldering iron or soldering station (T12 series or 936 series recommended), solder wire (recommended 63Sn37Pb), wire (28-30AWG wire recommended for signal wires, 14-18AWG recommended for power wires),35V 470uF capacitor (can be parallel soldered on the power input pad, optional), 10V 2A buck module (optional, required for DJI HD video transmission system, Caddx HD video transmission system, and some analog cameras and video transmission systems with poor filtering).

- Connecting the USB sub-board to the Flight Controller

- Wiring diagram for connection to USB sub-board

- Soldering order of wire to External TF Card module

- Wiring diagram for connection to TF sub-board

    - Precautions:

- Use silicone wires for signal lines. The flight controller terminal is of model SH1.0 and is small in size. When plugging or unplugging, support the rear of the socket with your hand to prevent it from falling off due to force.

- Interface sequence diagram

- Pin Definition of PWM Expansion Interface

| Pin Number | Pin Name | Pin Definition |
| --- | --- | --- |
| 1 | S12 | PWM_CH12 ESC/Servo |
| 2 | S11 | PWM_CH11 ESC/Servo |
| 3 | S10 | PWM_CH10 ESC/Servo |
| 4 | S9 | PWM_CH9 ESC/Servo |
| 5 | S8 | PWM_CH8 ESC/Servo |
| 6 | S7 | PWM_CH7 ESC/Servo |

- Soldering wire sequence of flight controller and servo expansion board

- Firmware

- For fixed-wing aircraft, ArduPlane and INAV firmware are recommended. ArduPlane firmware is pre-installed by default

- This flight controller is directly compatible with MatekF405-TE firmware.

- Latest stable firmware download links (updated 2025/11/09):

ArduPlane V4.6.3   MatekF405-TE

ArduCopter V4.6.3  MatekF405-TE

INAV 8.0.1  MatekF405TE_SD

BF        MatekF405-TE

The flight controller supports direct flashing via ground stations such as Mission Planner.

To access the flight controller configuration file, please click: MTKS F405TE_SD

- For the firmware flashing tutorial, please click：Bilibili Column — Ardupilot Fixed-Wing Flight Controller Firmware Flashing & Version Update

- For the Ardupilot firmware operation tutorial, please click：FlyingRC® F4Wing MK5User Manual

- (4)Wiring：

Complete wiring diagram for the Flight Controller

(Note: The 4IN1 75A ESC is the V3.2 version)

The 4-in-1 electronic control interface is exclusively designed for voltage detection and cannot provide power to the flight control system.

The 4IN1 75A ESC is currently available in two versions: V3.2 and V4.0, with differing wiring sequences. The V4.0 version complies with the BF wiring sequence standard. Customers should note the visual distinctions between the V3.2 and V4.0 models.

1.The 8P socket of the V4.0 version bears the printed warning: ‘Note the wire sequence.’

V3.2                    V4.0

2.Both versions bear the version number.

3.The V4.0 version bears an S/N code.

V3.2                       V4.0

- The Complete wiring diagram for the Flight Controller

- Wiring for Individual Equipment-Detailed Instructions

- ELRS Receiver

- The wiring diagram for connecting to the FlyingRC® ELRS receiver

- SBUS Receiver

- The wiring diagram for connecting to the SBUS receiver

- GPS

- The wiring diagram for connecting to the FlyingRC® GPS

- ！Note the GPS terminal pinout; please carefully examine the silk-screen markings on the GPS unit!

- Analogue Camera and Video Transmission Unit

      - The wiring diagram for connecting the Analogue Camera and the Video Transmission Unit

- 4IN1 ESC

- The wiring diagram for connecting to the FlyingRC® 4IN1 ESC

- (Note: The ESC is version V3.2)

- The wiring diagram for connecting to the FlyingRC® 4IN1 ESC

- (Note: The ESC is version V4.0)

- Digital Video Transmission Unit with SBUS (DJI,Walksnail)

Pay attention to the air unit voltage

- The wiring diagram for connecting to the Digital Video Transmission Unit with SBUS

- O4 BEC

- The wiring diagram for connection to the O4 BEC

- 40A Single ESC

- The wiring diagram for connecting the FlyingRC® AM32 Mini 40A ESC

      - （9） 75A Single ESC

- The wiring diagram for connecting to the FlyingRC® AM32 75A ESC

- !!! Please note that in the Ardupilot firmware, Serial numbers do not correspond one-to-one with UART/USART numbers. The mapping table is shown below !!!

- !!! Please note that after soldering, corresponding adjustments must be made to the Flight Controller Parameters. For instance, in the example diagram above, the GPS is connected to R5 and T5. In the corresponding serial port table below, R5 and T5 correspond to Serial3. Therefore, Serial3_Protocol must be set to GPS, and it must be confirmed that no other port functions are set to GPS!!!

- (5)I0 Interface & Serial Port Mapping & I2C Bus

- 1）PWM Output Functions

| PWM Group | IO耐压 | PWMs(建议功能) | GPIO对应编号 | Timer | DMA/DShot |
| --- | --- | --- | --- | --- | --- |
| Group1 | 5V | S1(Pin Header, Single ESC) | GPIO50 | TIM8_CH4 | DMA/DShot |
|  |  | S2(Pin Header, Single ESC) | GPIO51 | TIM8_CH3 | DMA/DShot |
| Group2 |  | S3(Pin Header, Servo) | GPIO52 | TIM1_CH3N | DMA/DShot |
|  |  | S4(Pin Header, Servo) | GPIO53 | TIM1_CH1 | DMA/DShot |
| Group3 |  | S5(Port, 4-in-1 ESC) | GPIO54 | TIM2_CH4 | DMA/DShot |
|  |  | S6(Port, 4-in-1 ESC) | GPIO55 | TIM2_CH3 | DMA/DShot |
|  |  | S7((Port, 4-in-1 ESC) | GPIO56 | TIM2_CH2 | DMA/DShot |
|  |  | S8(Port, 4-in-1 ESC) | GPIO57 | TIM2_CH1 | DMA/DShot |
| Group4 |  | S9(Pin Header, Servo) | GPIO58 | TIM12_CH1 | NO DMA |
| Group5 |  | S10(Pin Header, Servo) | GPIO59 | TIM13_CH1 | NO DMA |
| Group6 |  | S11(Solder Pad, Servo) | GPIO60 | TIM4_CH1 | NO DMA |
| Group7 |  | S12(Solder Pad, WS2812 LED Strip) | GPIO61 | TIM3_CH4 | DMA/DShot |
|  |  |  | SERVO12_FUNCTION 120, NTF_LED_TYPES neopixel |  |  |
| - LED pad is the S12 port on external PWM expansion board, can connect to WS2812 LEDs to show the state of the flight controller. - PWM channels under the same Timer Group can NOT be used simultaneously for ESCs using Dshot and Servos using 50Hz PWM. - S9-S11 does not have DMA, therefore could not be used for Dshot Protocol |  |  |  |  |  |

- 2）Serial Port Mapping List

|  | PCB Silkscreen | UART No. | Protocol | Recommended Configuration | SERIAL No. |
| --- | --- | --- | --- | --- | --- |
| UART 5V tolerant I/O | USB | USB |  | console | SERIAL0 |
|  | TX1 RX1 | USART1 | with DMA | MSP Displayport | SERIAL1 |
|  | TX3 RX3 | USART3 | NO DMA | USER | SERIAL2 |
|  | TX5 RX5 | UART5 | NO DMA | GPS | SERIAL3 |
|  | TX4 RX4 | UART4 | NO DMA | USER | SERIAL4 |
|  | TX6 RX6 | USART6 | TX6 with DMA | USER | SERIAL5 |
|  | TX2 RX2 SBUS | USART2 | with DMA | BRD_ALT_CONFIG 1 | SERIAL6 |
|  |  | SBUS pad | SBUS | BRD_ALT_CONFIG 0 Default |  |

- Correspondence between UART/USART and SERIAL in ArduPilot Firmware, and Their Default Functions

- For high-data-rate devices such as CRSF receivers and high-definition air units, it is recommended to connect them to DMA-enabled ports, such as UART1 (SERIAL1) or UART2 (SERIAL6).

- 3）I2C

| I2C |  | Config | Parameterl | Value |
| --- | --- | --- | --- | --- |
| I2C1 | 5V tolerant I/O | Compass | COMPASS_AUTODEC | 1 |
|  |  | onboard Baro SPL06 - 001 | Address | 0x76 |
|  |  | Digital Airspeed I2C | ARSPD_BUS | 1 |
|  |  | MS4525 | ARSPD_TYPE | 1 |
|  |  | DLVLR - L10D | ARSPD_TYPE | 9 |

- The Internal Baro is at 0x76, therefore, can NOT connect to external devices that are also on 0x76

- 4）ADC Analog Input

- -This product does not have an analog signal input interface.

- General Parameter Settings

- ArduPilot General Parameters：

- LOG_BACKEND_TYPE = 1，Enable TF card black box function to record flight logs

- BATT_VOLT_PIN     14

- BATT_VOLT_MULT   21.0

- BATT_CURR_PIN     15

- BATT_AMP_PERVLT  100

- INAV General Parameters：

- Default voltage meter ratio: 2100

- Default current meter ratio: 100

- Other FlyingRC® Products

| - FlyingRC® F4WSE Pro - F405 Fixed-Wing Flight Controller - Upgraded Version ¦ Enhanced Performance - Manual：https://www.kdocs.cn/l/cavm9EoVEq5A |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - FlyingRC® F4WSE MK1.5 - F405 Fixed-Wing Flight Controller - Classic model, unique design, highly acclaimed. - Manual：https://kdocs.cn/l/cpyd8MTS56DD |  |  |  |  |  |  |  |  |
| - FlyingRC® H7Wlite MK1 H743 Fixed-Wing Flight Controller - Flagship performance, full expansion, high-power BEC. - Manual：https://kdocs.cn/l/cpYaBiFKh8U1 |  |  |  |  |  |  |  |  |
| - FlyingRC® F4Wing Mini MK1 F405 Fixed-Wing Flight Controller - 2025 bestseller, ultra-compact, smallest on the market, great value. - Manual：https://kdocs.cn/l/cdUIdmpSpW7J |  |  |  |  |  |  |  |  |
| - FlyingRC® AM32 Dual ESC 40A 2-in-1 ESC - Unique design for dual-power aerial, land and water vehicles including drones. - Manual：https://www.kdocs.cn/l/cjwNtsKK6HeR |  |  |  |  |  |  |  |  |
| - FlyingRC® AM32 ESC V2.5 75A - Single ESC - Infineon gold-package MOSFETs, premium materials, superior heat dissipation. - Manual：https://kdocs.cn/l/cv6hUy67IdTJ |  |  |  |  |  |  |  |  |
| - FlyingRC® AM32 Mini ESC 40A V1.0 - Single ESC - Ultra-miniature, ideal for micro drones, high over-current capability, stable operation. - Manual：https://kdocs.cn/l/cqdLSysEyWFh |  |  |  |  |  |  |  |  |
| - FlyingRC® 10A 12S 440A Power Distribution Board for FPV Drones - Industry top choice, excellent quality and affordable price. - Manual：https://kdocs.cn/l/coel7OgrlCBP |  |  |  |  |  |  |  |  |
| - FlyingRC® H7D Pro MK1 H743 FPV Flight Controller with Dual Gyro - Fully Upgraded & More Powerful - Manual：https://www.kdocs.cn/l/ctcekCy676Ez |  |  |  |  |  |  |  |  |
| - FlyingRC® H7D MK1 H743 FPV Flight Controller with Dual Gyro - Powerful computing, stable and precise flight, rich interfaces, smooth control. - Manual：https://kdocs.cn/l/cqMh1I0RwAoD |  |  |  |  |  |  |  |  |
| - FlyingRC® F420 MK1 F405 FPV Flight Controller (20/30.5mm Mounting) - Dual mounting holes, all connectors, no soldering required, compliant with Betaflight interface standards. - Manual：https://kdocs.cn/l/ctGeLiN9ZYKj |  |  |  |  |  |  |  |  |
| - FlyingRC® 4-in-1 75A V3.2 Gold-Sealed FPV ESC - Infineon gold-package MOSFETs, superior heat dissipation & over-current capability, premium choice. - Manual：https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: H743 Flight Controller + 4-in-1 75A Gold-Sealed ESC - Professional flight choice, flagship materials & craftsmanship, reasonable price. - Manual：https://www.kdocs.cn/l/ctcekCy676Ez - https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: F405 Flight Controller + 4-in-1 75A Gold-Sealed ESC - Best-selling combo, excellent performance, affordable price. Manual：https://kdocs.cn/l/ctGeLiN9ZYKj https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: H743 Flight Controller + 4-in-1 45A ESC Winning combo: great performance, budget-friendly price Manual：https://www.kdocs.cn/l/ctcekCy676Ez https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: F405 Flight Controller + 4-in-1 45A ESC Hit combo, great value, budget-friendly Manual：https://kdocs.cn/l/ctGeLiN9ZYKj https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: F4WSE Pro - F405 Fixed-Wing Flight Controller + AM32 Dual ESC 40A 2-in-1 ESC Unique Design • Exclusive Offer • Bestselling Bundle Manual：https://www.kdocs.cn/l/cavm9EoVEq5A https://www.kdocs.cn/l/cjwNtsKK6HeR |  |  |  |  |  |  |
| - FlyingRC® ELRS 2.4G Diversity Receiver - True diversity reception, temperature compensation, high-sensitivity reception. - Manual：https://kdocs.cn/l/cbvlOlShmsPS |  |  |  |  |  |  |  |  |
| - FlyingRC® RM3100 SPI Module 罗盘模块 - Ultra-high Resolution • Low Power • Industry Top Choice - Manual：https://www.kdocs.cn/l/cdGuHvBVyndC |  |  |  |  |  |  |  |  |
| - FlyingRC® 10A 12S BEC Step-Down Module - Wide voltage input, high power, compact size, multiple voltages available, 10A continuous output. - Manual：https://kdocs.cn/l/clssNCKgNhLV |  |  |  |  |  |  |  |  |
| - FlyingRC® 5A BEC Step-Down Module - Ultra-small size, multiple voltages available, 5A continuous output. - Manual：https://kdocs.cn/l/cb9VdNywqCKA |  |  |  |  |  |  |  |  |
| - FlyingRC® Mini BEC For DJI O4 - Easy wiring, ultra-small size, stable power supply. - Manual：https://www.kdocs.cn/l/ckkebieur37K |  |  |  |  |  |  |  |  |
| - FlyingRC® CAN Bus Serial & PWM Expansion Board - Long-distance transmission, high speed, excellent stability. - Manual：https://www.kdocs.cn/l/cmxRyBnU4BXj |  |  |  |  |  |  |  |  |
| - FlyingRC® AM3 2ESC Programmer - Supports BL, BL32, AM32. Simple and easy to use. - Manual：https://kdocs.cn/l/cogBSEBe67mE |  |  |  |  |  |  |  |  |
| - FlyingRC® Digital Airspeed Sensor without Pitot Tube - Integrated design, smallest on the market. MS4525D protocol, I2C interface. - Manual：https://www.kdocs.cn/l/clPvEEEcJgCC |  |  |  |  |  |  |  |  |
| - FlyingRC® U-Blox M10 GPS - Compatible with all open-source flight controllers. - Manual：https://www.kdocs.cn/l/cdSYQrzx74O8 |  |  |  |  |  |  |  |  |