FlyingRC® H7D Pro MK1 H743 FPV Drone Flight Controller Product Manual

FlyingRC® is a globally-oriented professional brand of electronic devices. We specialize in the R&D and sales of drone electronics, core robot circuits and sensors. Our design team is based in Singapore, and our products are sold worldwide.

Since launched in 2023, FlyingRC® products have gained widespread recognition from customers as well as strong market response. As of April 2026, our cumulative sales volume has exceeded 46,000 units. Our products are widely applied in industries, robotics, emergency rescue, environmental protection & inspection, aerial photography, public security, retail and many other fields. Above all, we are committed to distinctive design and superior reliable quality.

Our Chief Technology Officer Liu Yuhan possesses outstanding expertise in circuit design. Over the past four years, he has designed more than 80 circuit projects and contributed 39 to open-source platforms. Moreover, he won the Design Award on the open-source platform X.JLC.com twice in 2023 and 2024, and has become the youngest certified designer on this platform.

He holds the NUS (National University of Singapore, rank 8th worldwide) Science and Technology Scholarship, and is currently majoring in Electronic Engineering with a focus on Aerospace Technology. In addition, all members of our technical team are graduates from prestigious universities with extensive professional product experience.

Flight Controller Overview：

- MCU STM32H743VIT6, Dual ICM-42688-P Gyroscopes, Barometer DPS310 or DPS368

- Analog/HD OSD, 7x UARTs (UART4 TX only, UART3 RX only), 13×PWM outputs (4 + 4 + 4 + 1), 1x I2C, 1x ADC (RSSI), onboard 9V BEC PinIO

- Black Box: onboard TF card slot

- 12~28V DC Input (3~6S LiPo; 5S/6S LiPo or 4S LiHV recommended)

- Dual BECs (MP9443 + MP9943): 5V/3A for Devices & 9V/3A for VTX and Camera

- Dimensions & Weight: 36.6mm×36.6mm×7.45mm, 9.3g

| Basic Parameters |  | Sensors |  |
| --- | --- | --- | --- |
| Model | FlyingRC® H7D Pro | IMU (Gyro & Accelerometer) | Dual Invensense 3rd Gen ICM-42688-P |
| Dimensions | 36.6mm*36.6mm*7.45mm | Barometer | DPS310/ DPS368 |
| Mounting Hole Spacing | 30.5mm | Magnetometer | No Onboard Magnetometer |
| Weight | 9g | Analog OSD | AT7465E |
| MCU |  | Power Output |  |
| MCU | STM32H743VIT6 | BEC Chip Model | MP9443 + MP9943 |
| Clock Speed | 480 MHz | Onboard BEC (for Devices) | 5V3A |
| Flash | 2 MB | Onboard BEC (for VTX) | 9V3A |
| RAM | 1 MB |  |  |
| Interfaces |  | Firmware Support |  |
| UART | 7×UARTs | Betaflight | Supported (Factory Default) |
|  | UART1,UART2, UART3: RX only | INAV | Supported |
|  | UART4-7, UART8: TX only | Ardupilot | Supported |
| PWM | 13 channels (including 1 LED channel) | PX4 | Not Supported |
| I2C | 1 |  |  |
| Current ADC Sampling | Supported |  |  |
| SWD Debug | Not Supported | Operating Environment |  |
| Buzzer Port | Yes, supports passive buzzer | VBAT Input Range | 12-28V DC IN 3-6S LiPo |
| LED Strip Port | Supports WS2812 | Power Input | Same as VBAT Input Range |
| USB-TYPE-C | Onboard, direct plug-in | Operating Temperature | -10-100℃ |
| Black Box Storage | onboard TF card slot | Storage Temperature | 0-40℃ |
| SBUS | Built-in inverter on H7 MCU |  |  |
|  | SBUS can be connected to any RX port |  |  |

PackageContents:

① 1× FlyingRC H7D H743 Pro Flight Controller

② 4× Silicone Anti-Vibration Standoffs

③ 1× DJI VTX 6-Pin Cable (15cm)

④ 3× SH1.0 4-Pin 30AWG Ultra-Flexible Silicone Cable (10cm)

⑤ 1× SH1.0 3-Pin 30AWG Ultra-Flexible Silicone Cable (10cm)

⑥ 1× SH1.0 6-Pin 30AWG Ultra-Flexible Silicone Cable (10cm)

⑦ 1× SH1.0 8-Pin Crimp Terminal

⑧ 1× SH1.0 8-Pin 30AWG Ultra-Flexible Silicone Wire (3.5cm)

Features:

- MCU STM32H743VIT6, 2MB Flash, 1MB RAM, 480MHz, delivering powerful performance.

- Dual ICM-42688-P (Gyroscopes & Accelerometers), optimized for drones to ensure high stability.

- 30.5mm mounting pattern, standard M4 threaded holes (compatible with M3 screws with anti-vibration standoffs), suitable for 4–12 inch FPV drones.

- 13 PWM Outputs: Supports simultaneous connection of dual 4-in-1 ESCs, suitable for X8 or 8-axis configurations.

- Reserved interfaces for DJI VTX, CAN, GPS, LED, ESCs, receivers, etc., with configurable UART protocols for flexible expansion. Solder pads for camera, analog VTX, UART1, and power (VBAT/GND) are also reserved.

- Equipped with dual high-precision gyroscopes and a high-precision barometer, delivering significantly improved stability compared to comparable flight controllers (especially noticeable when using AP firmware).

- Robust BEC Output: 5V/3A for device power supply, with the strongest current capacity in its size category.

- Switchable Onboard 9V BEC: Controlled via PINIO1 (User1 in BF firmware), preventing VTX from overheating and damage during bench testing.

- Onboard TF card slot: Plenty of storage for multiple flight data recordings.

LAYOUT：

3D schematic diagram of the Flight Controller - front view

3D schematic diagram of the Flight Controller - back view

Schematic diagram of connector and pad functions - front view

Schematic diagram of connector and pad functions - back view

Preparations Before Use：

‼️CRITICAL‼️

Make an 8P ESC connection cable with the correct wiring sequence according to the ESC model. If you have any questions about the wiring sequence, please consult in the official FlyingRC® group. Incorrect wiring may cause the Flight Controller to burn out!

‼️CRITICAL‼️

Wiring diagram (applicable to BF, AP, and INAV firmwares)：

The functions of the Flight Controller Pads are clearly labeled with text directly on the board. For example, T4 represents the UART4 TX port, and R1 represents the USART1 RX port.

- 4IN1 75A Gold-Sealed ESC

Wiring Diagram for Connection with FlyingRC® 4IN1 75A Gold-Sealed ESC

(Note: The ESC is V4.0 version)

- ELRS

- Note：the TX/RX cross connection! (FC TX → Receiver RX, FC RX → Receiver TX)

- If using non-SBUS/PPM protocol receivers (e.g. ELRS) on UART6 (Serial7, silkscreen: SBUS&CRSF), set brd_alt_config = 1.

- Power off and restart the device after configuration!

Wiring Diagram for Connection with FlyingRC® ELRS

- GPS

Wiring Diagram for Connection with FlyingRC® GPS

- Digital Video Transmitter

Wiring Diagram for Connection with DJI Digital Video Transmitter

- （5）Buzzer and LED

- Wiring Diagram for Connection with Buzzer and LED

- Flight Controller Firmware Flashing

BF, AP, and INAV firmwares are recommended. The Flight Controller comes pre-flashed with BF4.4.2 firmware as factory default.  The following wiring instructions will use BF firmware as an example.

Firmware is officially compiled by FlyingRC® and not available in the public Betaflight firmware list. Please download the firmware from our official website.

Firmware Target：FLRC H7D

Betaflight Firmware Flashing Tutorial：Bilibili Article — Ardupilot Fixed-Wing: Flight Controller Firmware Flashing & Version Updates

Technical Support Notes:

- Limited technical support is provided for BF and Ardupilot firmwares.

- No technical support is available for INAV firmware.

Detailed Explanation of Flight Controller Interfaces：

- Serial Ports (UARTx, Serialx)

️NOTE: In Ardupilot firmware, the Serial port numbers do not correspond with UART/USART numbers. The mapping table is shown below. ‼️

| PCB silkscreen |  |  | UART No. | Configration | SERIAL_X |
| --- | --- | --- | --- | --- | --- |
| USB | PA11/PA12 | 5 V tolerant I/O | USB | console | SERIAL0 |
| RX7 TX7 RTS7 CTS7 | PE7/8/9/10 | 3.3 V tolerant I/O | UART7 | telem1 | SERIAL1 |
| TX1 RX1 | PA9/PA10 | 5 V tolerant I/O | USART1 | telem2 | SERIAL2 |
| TX2 RX2 | PD5/PD6 | 5 V tolerant I/O | USART2 | GPS1 | SERIAL3 |
| TX3 RX3 | PD8/PD9 | 5 V tolerant I/O | USART3 | GPS2 | SERIAL4 |
| TX8 RX8 | PE1/PE0 | 5 V tolerant I/O | UART8 | USER | SERIAL5 |
| TX4 RX4 | PB9/PB8 | 5 V tolerant I/O | UART4 | USER | SERIAL6 |
| TX6 RX6 | PC6/PC7 | 5 V tolerant I/O | USART6 | RC input/Receiver | SERIAL7 |

Ardupilot Serial Port Mapping Table

Note: After wiring, the Flight Controller Parameters must be adjusted accordingly. For example, in the sample Wiring Diagram above, the GPS is connected to R2 and T2. In the Serial Port Mapping Table below, R2 and T2 correspond to Serial3. Therefore, set Serial3_Protocol to GPS and ensure no other ports are configured for GPS functionality.

If you use a receiver that uses non-SBUS/PPM protocols (such as ELRS) on Serial7, you need to set brd_alt_config to 1.

- （2）PWM output function

| PWM Group | PWM 通道 |  |  | GPIO | Timer |
| --- | --- | --- | --- | --- | --- |
| Group1 | S1 | PB0 | 5V tolerant I/O | PWM1 GPIO50 | TIM8_CH2N |
|  | S2 | PB1 | 3.3V tolerant I/O | PWM2 GPIO51 | TIM8_CH3N |
| Group2 | S3 | PA0 | 5V tolerant I/O | PWM3 GPIO52 | TIM5_CH1 |
|  | S4 | PA1 | 5V tolerant I/O | PWM4 GPIO53 | TIM5_CH2 |
|  | S5 | PA2 | 5V tolerant I/O | PWM5 GPIO54 | TIM5_CH3 |
|  | S6 | PA3 | 5V tolerant I/O | PWM6 GPIO55 | TIM5_CH4 |
| Group3 | S7 | PD12 | 5V tolerant I/O | PWM7 GPIO56 | TIM4_CH1 |
|  | S8 | PD13 | 5V tolerant I/O | PWM8 GPIO57 | TIM4_CH2 |
|  | S9 | PD14 | 5V tolerant I/O | PWM9 GPIO58 | TIM4_CH3 |
|  | S10 | PD15 | 5V tolerant I/O | PWM10 GPIO59 | TIM4_CH4 |
| Group4 | S11 | PE5 | 5V tolerant I/O | PWM11 GPIO60 | TIM15_CH1 |
|  | S12 | PE6 | 5V tolerant I/O | PWM12 GPIO61 | TIM15_CH2 |
| Group5 | LED | PA8 | 5V tolerant I/O | PWM13 GPIO62 | TIM1_CH1 |
|  |  |  |  | SERVO13_FUNCTION 120, NTF_LED_TYPES neopixel |  |

PWM Output Function Table

The LED pad is the S13 port, which can connect to a WS2812 light strip and be freely configured in the BF ground station (AP firmware displays the Flight Controller status).

PWM ports in the same Group cannot be used for Dshot ESCs and 50Hz PWM servos simultaneously.

- （3）I2C Bus

| I2C num |  |  | Config | Parameterl | Value |
| --- | --- | --- | --- | --- | --- |
| I2C1 CL1/DA1 | PB6/PB7 | 5V tolerant I/O | Compass | COMPASS_AUTODEC | 1 |

I2C bus Table

- ADC Analog Signal Input

| Pin |  |  | Definition | Config | Value |
| --- | --- | --- | --- | --- | --- |
| Vbat pad 1K:10K divider builtin | PC0 | 0 - 36V | Vbat ADC onboard battery voltage sense | BATT_VOLT_PIN BATT_VOLT_MULT | 10 11.0 |
| Curr pad | PC1 | 0 - 3.3V | Current ADC onboard current sense | BATT_CURR_PIN BATT_AMP_PERVLT | 11 66.7 |
| CU2 Pad | PA7 | 0 - 3.3V | Current2 ADC | BATT2_CURR_PIN BATT2_AMP_PERVLT | 7 / |

ADC Analog Signal Input Table

Betaflight General Parameter Settings：

Gyroscope Installation Orientation: CW90Flip

Ardupilot General Parameter Settings：

LOG_BACKEND_TYPE = 4，Enable TF card black box function to record flight logs

BATT_VOLT_PIN     10

BATT_VOLT_MULT   21.0，Default voltmeter ratio (5% error), additional calibration not required

BATT_CURR_PIN     11

INAV General Parameter Settings：

Default voltmeter ratio: 21000

- Other FlyingRC® Products

| - FlyingRC® F4WSE Pro - F405 Fixed-Wing Flight Controller - Upgraded Version ¦ Enhanced Performance - Manual：https://www.kdocs.cn/l/cavm9EoVEq5A |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - FlyingRC® F4WSE MK1.5 - F405 Fixed-Wing Flight Controller - Classic model, unique design, highly acclaimed. - Manual：https://kdocs.cn/l/cpyd8MTS56DD |  |  |  |  |  |  |  |  |
| - FlyingRC® H7Wlite MK1 H743 Fixed-Wing Flight Controller - Flagship performance, full expansion, high-power BEC. - Manual：https://kdocs.cn/l/cpYaBiFKh8U1 |  |  |  |  |  |  |  |  |
| - FlyingRC® F4Wing Mini MK1 F405 Fixed-Wing Flight Controller - 2025 bestseller, ultra-compact, smallest on the market, great value. - Manual：https://kdocs.cn/l/cdUIdmpSpW7J |  |  |  |  |  |  |  |  |
| - FlyingRC® AM32 Dual ESC 40A 2-in-1 ESC - Unique design for dual-power aerial, land and water vehicles including drones. - Manual：https://www.kdocs.cn/l/cjwNtsKK6HeR |  |  |  |  |  |  |  |  |
| - FlyingRC® AM32 ESC V2.5 75A - Single ESC - Infineon gold-package MOSFETs, premium materials, superior heat dissipation. - Manual：https://kdocs.cn/l/cv6hUy67IdTJ |  |  |  |  |  |  |  |  |
| - FlyingRC® AM32 Mini ESC 40A V1.0 - Single ESC - Ultra-miniature, ideal for micro drones, high over-current capability, stable operation. - Manual：https://kdocs.cn/l/cqdLSysEyWFh |  |  |  |  |  |  |  |  |
| - FlyingRC® 10A 12S 440A Power Distribution Board for FPV Drones - Industry top choice, excellent quality and affordable price. - Manual：https://kdocs.cn/l/coel7OgrlCBP |  |  |  |  |  |  |  |  |
| - FlyingRC® H7D Pro MK1 H743 FPV Flight Controller with Dual Gyro - Fully Upgraded & More Powerful - Manual：https://www.kdocs.cn/l/ctcekCy676Ez |  |  |  |  |  |  |  |  |
| - FlyingRC® H7D MK1 H743 FPV Flight Controller with Dual Gyro - Powerful computing, stable and precise flight, rich interfaces, smooth control. - Manual：https://kdocs.cn/l/cqMh1I0RwAoD |  |  |  |  |  |  |  |  |
| - FlyingRC® F420 MK1 F405 FPV Flight Controller (20/30.5mm Mounting) - Dual mounting holes, all connectors, no soldering required, compliant with Betaflight interface standards. - Manual：https://kdocs.cn/l/ctGeLiN9ZYKj |  |  |  |  |  |  |  |  |
| - FlyingRC® 4-in-1 75A V3.2 Gold-Sealed FPV ESC - Infineon gold-package MOSFETs, superior heat dissipation & over-current capability, premium choice. - Manual：https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: H743 Flight Controller + 4-in-1 75A Gold-Sealed ESC - Professional flight choice, flagship materials & craftsmanship, reasonable price. - Manual：https://www.kdocs.cn/l/ctcekCy676Ez - https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: F405 Flight Controller + 4-in-1 75A Gold-Sealed ESC - Best-selling combo, excellent performance, affordable price. Manual：https://kdocs.cn/l/ctGeLiN9ZYKj https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: H743 Flight Controller + 4-in-1 45A ESC Winning combo: great performance, budget-friendly price Manual：https://www.kdocs.cn/l/ctcekCy676Ez https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: F405 Flight Controller + 4-in-1 45A ESC Hit combo, great value, budget-friendly Manual：https://kdocs.cn/l/ctGeLiN9ZYKj https://kdocs.cn/l/ceLilc8Gaa21 |  |  |  |  |  |  |
| - FlyingRC® Stack Combo: F4WSE Pro - F405 Fixed-Wing Flight Controller + AM32 Dual ESC 40A 2-in-1 ESC Unique Design • Exclusive Offer • Bestselling Bundle Manual：https://www.kdocs.cn/l/cavm9EoVEq5A https://www.kdocs.cn/l/cjwNtsKK6HeR |  |  |  |  |  |  |
| - FlyingRC® ELRS 2.4G Diversity Receiver - True diversity reception, temperature compensation, high-sensitivity reception. - Manual：https://kdocs.cn/l/cbvlOlShmsPS |  |  |  |  |  |  |  |  |
| - FlyingRC® RM3100 SPI Module 罗盘模块 - Ultra-high Resolution • Low Power • Industry Top Choice - Manual：https://www.kdocs.cn/l/cdGuHvBVyndC |  |  |  |  |  |  |  |  |
| - FlyingRC® 10A 12S BEC Step-Down Module - Wide voltage input, high power, compact size, multiple voltages available, 10A continuous output. - Manual：https://kdocs.cn/l/clssNCKgNhLV |  |  |  |  |  |  |  |  |
| - FlyingRC® 5A BEC Step-Down Module - Ultra-small size, multiple voltages available, 5A continuous output. - Manual：https://kdocs.cn/l/cb9VdNywqCKA |  |  |  |  |  |  |  |  |
| - FlyingRC® Mini BEC For DJI O4 - Easy wiring, ultra-small size, stable power supply. - Manual：https://www.kdocs.cn/l/ckkebieur37K |  |  |  |  |  |  |  |  |
| - FlyingRC® CAN Bus Serial & PWM Expansion Board - Long-distance transmission, high speed, excellent stability. - Manual：https://www.kdocs.cn/l/ckkebieur37K |  |  |  |  |  |  |  |  |
| - FlyingRC® AM3 2ESC Programmer - Supports BL, BL32, AM32. Simple and easy to use. - Manual：https://kdocs.cn/l/cogBSEBe67mE |  |  |  |  |  |  |  |  |
| - FlyingRC® Digital Airspeed Sensor without Pitot Tube - Integrated design, smallest on the market. MS4525D protocol, I2C interface. - Manual：https://www.kdocs.cn/l/clPvEEEcJgCC |  |  |  |  |  |  |  |  |
| - FlyingRC® U-Blox M10 GPS - Compatible with all open-source flight controllers. - Manual：https://www.kdocs.cn/l/cdSYQrzx74O8 |  |  |  |  |  |  |  |  |